# Specifications

| COMPONENT | TYPE | MATERIAL | WHY |
| --- | --- | --- | --- |
| Frame | Track Racing | Aluminium | Excellcent performance, easy to pedal, light weight, strong and durable. |
| Fork | Ahead 1-1/8 inch - 43mm Rake | Hi-Tensile Steel | Excellcent road vibration absorbtion, reliable and durable, rake chosen for it's 'feel' on the road for predictable handling |
| Headset | Ahead 1-1/8 inch - open cage ball bearing system | Steel cups | Quick and easy maintenance, could be improved with a sealed cartridge system - chosen for price |
| Handlebar | 1 inch riser handlebar | Aluminium | Light weight, strong, corrosion resistant |
| Stem | 90 mm - 10 degree rise | " | " |
| Grips | 125 mm | Kraton Rubber | Comfort, shock absorbtion and firm grip |
| Brake Levers | Caliper Levers for road brakes | Aluminium | Light weight, strong, corrosion resistant and stiff transfer of power for powerful and reliable braking |
| Brakes | 57 mm Drop Dual Pivot Calipers | " | Light weight, strong, corrosion resistant and effective powerful braking thanks the the dual pivot design |
| Cables | Generic MTB/Hybrid Brake | Zinc alloy Steel | Nothing special - could be improved with Stainless Steel cables |
| Housing | Lined slick quality housing | Nylon and Steel | Lined housing for easy use of brakes |
| Seatpost | Silver 27.2 mm Single bolt clamp | Aluminium | Light weight, strong, corrosion resistant |
| Saddle | 155 mm Wide Comfort Saddle | Nylon, steel, dense foam | For comfort, but not too bulky or heavy |
| Pedals | Dual-sided, symmetrical all Metal pedals - 9/16 standard | Steel and Aluminium | For strength and reliability |
| WHEELS | --------------------------------- | ---------------- | ------------------------------------------------------------------ |
| Tyres | 700 x 25c - City, all weather tread pattern | Mid-density rubber | Long-lasting, durable with a decent grip and performance maximum pressure 50 - 60 psi (though the sidewall says 95 psi - highly recommend replacing these with folding bead tyres as they fit the wheels much better, otherwise you can not go to high pressures safely. |
| Tubes | Generic tubes | Butyl rubber | Moderate Reliability - improve with expensive after market tubes |
| Rim Tape | Generic High Pressure rim tape | Plastic | " |
| Rims | 700x18c - 30mm depth - 32hole | Aluminium | Excellent strength to weight, stiff power transfer with aerodynamic qualities to slice through the air as they rotate, creates a wheel that is very strong and hard to damage |
| Hubs | Standard cone and ball bearing track hubs - front 100mm, rear 120mm with dual threading - 32hole | Aluminium body, steel axles and bearings | Reliability and ease of maintenance, no fancy, easily damaged cartridge bearings. Recommend regular maintenance however to keep these healthy. - Upgrade would mean new wheels which cost three times as much and have very few added benefits |
| Spokes | Generic strong 2.0mm spokes | Stainless Steel | Strength and reliabilty |
| DRIVETRAIN | --------------------------------- | ---------------- | ------------------------------------------------------------------ |
| Chainset | 42 T Front Chainring weld mounted on light weight crank arms | Steel Chainring, Aluminium Crank Arms | Light weight, durable mid-range gearing for ease of use. Want something faster / harder to pedal? You would be looking at a new chainset which cost at least £20. |
| Bottom Bracket | Generic Cup and Axle | Steel | Not ideal - upgrade with a sealed cartidge unit from Shimano, prices vary - as I write this, I am trying to get the bikes to come stock with sealed bottom bracket units - chosen solely to keep price low |
| Chain | Generic 1/8 inch chain | Steel | Reliable and strong, keep it clean, oiled and happy |
| Cog - Fixed - Side 1 | 16 Tooth with lockring | Steel | Very strong, reliable and long-lasting, change the chain regularly to keep this part lasting even longer |
| Freewheel - Side 2 | Generic 16 Tooth | Steel | Questionable and uneven performance, if yours doesn't work send it to me for a replacement - chosen to keep the price low |